Adding Custom IP to the System

Introduction

This lab guides you through the process of creating and adding a custom peripheral to a processor system by using the Vivado IP Packager. You will create an AXI4Lite interface peripheral.

# Objectives

After completing this lab, you will be able to:

Use the IP Packager feature of Vivado to create a custom peripheral

Modify the functionality of the IP

Add the custom peripheral to your design

Add pin location constraints

Add block memory to the system

Procedure

This lab is separated into steps that consist of general overview statements that provide information on the detailed instructions that follow. Follow these detailed instructions to progress through the lab.

This lab comprises 4 primary steps: You will use a peripheral template to create a peripheral, Package the IP using IP Packager, import, add and connect the IP in the design, and add the Block RAM (BRAM) Memory.

Design Description

You will extend the Lab 2 hardware design by creating and adding an AXI peripheral (refer to LED_IP in Figure 1) to the system, and connecting it to the LEDs on the ZYBO board.  You will use the IP Packager to generate the custom IP.    Next, you will connect the peripheral to the system and add pin location constraints to connect the LED display controller peripheral to the on-board LED display.  Finally, you will add BRAM Controller and BRAM before generating the bitstream.

Figure 1. Design Updated from Previous Lab

General Flow for this Lab

Step 1: Create Custom IP using

Peripheral Template

Step 2: Package Peripheral using IP Packager

Step 3:

Set the Project Settings

Step 4:

Add the IP. BRAM, and Constraints

Create a Custom IP using the Create and Package IP Wizard	Step 1

Use the provided axi_lite slave peripheral template and the custom IP source code to create a custom IP.

Open Vivado by selecting Start > All Programs > Xilinx Design Tools > Vivado 2013.4 > Vivado 2013.4

Click Manage IP and select New IP Location and click Next in the New IP Location window

Select Verilog as the Target Language, Mixed as the Simulator language, and for IP location, type C:\xup\embedded\labs\led_ip and click Finish (leave other settings as defaults)

Figure 2. New IP Location form

A Virtex 7 part is chosen for this project, but later compatibility for other devices will be added to the packaged IP later.

Click OK to create the led_ip directory.

Run the Create and Package IP Wizard

Select Tools > Create and Package IP

In the window, click Next

Select Create new AXI4 peripheral, specify the IP Definition location as C:/xup/embedded/labs/led_ip and click Next

Fill in the details for the IP

- Name: led_ip

- Display Name: led_ip_v1_0

- (Fill in a description, Vendor Name, and URL)

Figure 3. Updating Peripheral Details form

Click Next

Change the Name of the interface to S_AXI

Leave the other settings as default and click Next (Lite interface, Slave mode, Data Width 32, Registers 4)

Figure 4. Naming the AXI interface

Select Generate Drivers and click Next

Select Add IP to Catalog and open IP in editing session selected and click Finish

In the sources panel, double-click the led_ip_v1_0.v file and scroll down to line 15 where users port definition space is provided.

Add the line:

- output wire [3:0]      LED,

- (Notice the extra comma needed at the end of line)

Figure 5. Adding users port definition

Insert the following at line ~48:

- . LED(LED),

Figure 6. Adding port connection with a lower-level module

Save the file by selecting File > Save File

Expand led_ip_v1_0 in the sources view if necessary, and open led_ip_v1_0_S_AXI.v

Add the LED port to this file too, at line 15

Figure 7. Declaring users port in the lower-level module

Scroll down to ~line 397 and insert the following code to instantiate the user logic for the LED IP

- (This code can be typed directly, or copied from the user_logic_instantiation.txt file in the lab3 source folder.)

Figure 8. Instantiating lower-level user module

Check all the signals that are being connected and where they originate.

Save the file by selecting File > Save File

Click on the Add Sources in the Flow Navigator pane, select Add or Create Design Sources, browse to c:\xup\embedded\sources\lab3, select the lab3_user_logic.v file and click OK, and then click Finish to add the file.

Check the contents of this file to understand the logic that is being implemented.  Notice the formed hierarchy.

Click Run Synthesis and Save if prompted. (This is to check the design synthesizes correctly before packaging the IP.

Check the Messages tab for any errors and correct if necessary before moving to the next step

When Synthesis completes successfully, click Cancel.

It is important that all source files that the IP will use be located under the IP project.

Update the IP project with the necessary source files in the appropriate directory

Expand the source hierarchy, if needed, and select lab3_user_logic entry.

Right-click and select Remove File from the Project.

Click OK to remove the file.

Similarly, remove all the files which were added from outside the project directory.  In this lab, you don’t have more files to be removed.

Using Windows Explorer, copy the lab3_user_logic.v file from the c:\xup\embedded\sources\lab3 directory and paste it in the current IP project directory at c:\xup\embedded\labs\led_ip\led_ip_1.0\hdl directory.

Click on the Add Sources in the Flow Navigator pane, select Add or Create Design Sources, browse to c:\xup\embedded\labs\led_ip\led_ip_1.0\hdl, select the lab3_user_logic.v file and click OK, and then click Finish to add the file.

Make sure that you select the file from the IP project directory and not the sources\lab3 directory.

Package the IP

Click on the Package IP – led_ip tab

For the IP to appear in the IP catalog in particular categories, the IP must be configured to be part of those categories. To change which categories the IP will appear in the IP catalog click the browse box on the Categories line. This opens the Choose IP Categories window

For the exercise purpose, uncheck the AXI Peripheral box and check the Basic Elements and click OK.

Figure 9. Specify the category for IP Packager IP

Select IP Compatibility. This shows the different Xilinx FPGA Families that the IP supports. The value is inherited from the device selected for the project.

Right click in the Family Support table and select Add Family… from the menu.

Select the Zynq family as we will be using this IP on the ZYBO board, and click OK.

You can also customize the address space and add memory address space using the IP Addressing and Memory category.  We won’t make any changes.

Click on IP File Groups and click Merge changes from IP File Groups Wizard

Figure 10. Updating the file group

This is to update the IP Packager with the changes that were made to the IP and the lab3_user_logic.v file that was added to the project.

Do the same for IP Customization Parameters (Merge changes from IP Customization Parameters Wizard)

Notice that the IP Ports view now shows the user created LED port

Figure 11. User port imported

Select Review and Package, and notice the path where the IP will be created.

Figure 12. IP location

Click Re-Package IP

You will see a warning message indicating that the led_ip directory is already in the project. Click OK to ignore it.

In the Vivado window click File > Close Project

Modify the Project Settings	Step 3

Open the previous project, or use the lab2 project from the labsolution directory, and save the project as lab3. Set Project Settings to point to the created IP repository.

Start the Vivado if necessary and open either the lab2 project you created in the previous lab or the lab2 project in the labsolution directory

Select File > Save Project As … to open the Save Project As dialog box. Enter lab3 as the project name.  Make sure that the Create Project Subdirectory option is checked, the project directory path is c:\xup\embedded\labs\ and click OK.

This will create the lab3 directory and save the project and associated directory with lab3 name.

Click Project Settings in the Flow Navigator pane.

Select IP in the left pane of the Project Settings form.

Click on the Add Repository… button, browse to c:\xup\embedded\labs\led_ip and click Select.

The led_ip_v1_0 IP will appear the IP in the Selected Repository window.

Figure 13. Specify IP Repository

Click OK.

Add the Custom IP, BRAM, and the Constraints 	Step 4

Add led_ip to the design and connect to the AXI4Lite interconnect in the IPI.  Make internal and external port connections.   Establish the LED port as external FPGA pins.

Click Open Block Design under IP Integrator in the Flow Navigator pane, and select system.bd to open IP Integrator

Click the Add IP icon  and search for led_ip_v1_0 in the catalog by typing “led” in the search field.

Figure 12. Searching for led_ip in the IP Catalog

Double-click led_ip_v1_0 to add the core to the design.

Select the IP in the block diagram and change the instance name to led_ip in the properties view.

Click on Run Connection Automation, select /led_ip/S_AXI and click OK to automatically make the connection from the AXI Interconnect to the IP.

Click the regenerate button () to redraw the diagram.

Figure 13. LED IP Block added and connected

Select the LED[3:0] port on the led_ip instance (by clicking on its pin), right-click and select Make External.

Figure 14. LED external port added and connected

Select the Address Editor tab and verify that an address has been assigned to led_ip.

Figure 15. Address assigned for led_ip

Add BRAM to the design

In the Block Diagram, click the Add IP icon  and search for BRAM and add one instance of the AXI BRAM Controller

Run Connection Automation on axi_bram_ctrl_0/S_AXI and click OK when prompted to connect it to the M_AXI_GP0 Master.

Double click on the block to customize it and change the number of BRAM interfaces to 1 and click OK.

Notice that the AXI Protocol being used is AXI4 instead of AXI4Lite since BRAM can provide higher bandwidth and the controller can support burst transactions.

Figure 16. Customize BRAM controller

Run Connection Automation to add and connect a Block Memory Generator (This could be added manually)

Validate the design to ensure there are no errors, and click the regenerate button () to redraw the diagram.

- The design should look similar to the figure below.

Figure 17. Completed Block Diagram

In the Address editor, increase the Range of the axi_bram_ctrl_0 to 8K

Figure 20. Adjusting memory size

Press F6 to validate the design one last time.

Update the top-level wrapper and add the provided lab3_system.xdc constraints file.

In the sources view, right-click on the block diagram file, system.bd, and select Create HDL Wrapper to update the HDL wrapper file, and when prompted, select Let Vivado Manage Wrapper and auto-update and click OK

The system_wrapper.vhd file will be updated to include the new IP and ports.  Double-click on the system-wrapper content to verify that LED port has been added.

Click Add Sources in the Flow Navigator pane, select Add or Create Constraints, and click Next.

Click the Add Files button, browse to the c:\xup\embedded\sources\lab3 folder, select lab3_system.xdc

Click Finish to add the file.

Expand Constraints folder in the Sources pane, and double-click lab3_system.xdc file entry to see its content. This file contains the pin locations and IO standards for the LEDs on the ZYBO board. This information can usually be found in the manufacturer’s datasheet for the board.

Click on the Generate Block Design > system.bd in the Flow Navigator, to generate the output products of various IPs added to the project.

Select File > Close Project to close the project as we will implement the design in the next lab.

Click Save, and OK to save the design and close the project.

Conclusion

Vivado IP packager was used to import a custom IP block into the IP library. The IP block was then added to the system. Connection automation was run where available to speed up the design of the system by allowing Vivado to automatically make connections between IP. An additional BRAM was added to the design. Finally, pin location constraints were added to the design.